DESIGN TASK

ECA5604-5G/6G COMMUNICATION SYSTEMS

NAME: V Sadhana

REG NO: 192412242

17. LEO Satellite-Assisted Communication Model

Aim: To model a LEO satellite-assisted wireless communication system and analyze the variation of received signal strength with distance.

Matlab Code:

clc;

clear;

close all;

% LEO Satellite Communication Model

h = 550;                 % LEO altitude (km)

R = 6371;                % Earth radius (km)

fc = 2e9;                % Frequency (Hz)

c = 3e8;                 % Speed of light (m/s)

Pt = 30;                % Transmit power (dBm)

Gt = 20;                % Transmitter antenna gain (dBi)

Gr = 20;                % Receiver antenna gain (dBi)

% Ground coverage distance

d = linspace(550,1500,100);       % km

d_m = d*1000;

% Free-space path loss

FSPL = 20*log10(d_m) + 20*log10(fc) ...

+ 20*log10(4*pi/c);

% Received power

Pr = Pt + Gt + Gr - FSPL;

% Plot

plot(d,Pr,'LineWidth',2);

grid on;

xlabel('Satellite-Ground Distance (km)');

ylabel('Received Power (dBm)');

title('LEO Satellite-Assisted Communication');

Output:

Result: The graph shows that received power decreases as the LEO satellite-ground distance increases, demonstrating the effect of free-space path loss.

18. Comparative Architecture of LEO, MEO and GEO Backhaul Systems

Aim: To compare LEO, MEO and GEO satellite backhaul systems based on propagation delay.

Matlab Code:

clc;

clear;

close all;

% Satellite altitudes

altitude = [550 10000 35786];    % km

% Speed of light

c = 3e5;                         % km/s

% Approximate one-way propagation delay

delay = altitude/c*1000;         % ms

% Plot

bar(delay);

grid on;

set(gca,'XTickLabel',{'LEO','MEO','GEO'});

xlabel('Satellite Type');

ylabel('One-Way Propagation Delay (ms)');

title('Comparison of LEO, MEO and GEO Backhaul Systems');

% Display values

disp('Satellite   Altitude(km)   Delay(ms)');

disp([altitude' delay']);

Output:

Result: The graph compares propagation delay. LEO has the lowest delay, MEO has moderate delay, and GEO has the highest delay because of its much greater orbital altitude.

Quick comparison

| Satellite | Approx. Altitude | Relative Delay | Coverage |
| --- | --- | --- | --- |
| LEO | 550 km | Very Low | Small |
| MEO | 10,000 km | Medium | Medium |
| GEO | 35,786 km | High | Very Large |